SIMATS ENGINEERING

ASSESSMENT TOOL 2

COURSE NAME: COMPUTER NETWORKS COURSE CODE: CSA0713

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 30%

Annexure A

Scenario-Based

Code of Conduct:

I 	 (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

Annexure A

Scenario-Based

- A newly established Smart Campus is deploying a secure network infrastructure for its academic and administrative departments. The network administrator has been allocated the IP address block 192.168.100.0/24. Based on the expected number of devices, the following subnet masks have already been assigned:

School of Computing → /25 School of Electronics → /26

School of Mechanical Engineering → /27

As the network engineer, design the subnet allocation by performing the following tasks:

- Determine	the	network	address	and	broadcast	address	for	each department.

- A.Network Address 192.168.100.00

- Identify the valid host IP address range for each subnet.

A. Broadcast adress: 192.168.100.127

- Calculate the number of usable host addresses available in each subnet.

A. Host range : 192.168.100.192.168.100.126

- Determine the remaining unused IP address range within the given network block.

A.Usable Hosts:126

- A multinational software company is expanding its headquarters and has been assigned the private IP block 10.10.0.0/16. Different departments require different subnet sizes based on the number of employees. The network administrator has allocated the following subnet masks:

Software Development → /24 Quality Assurance → /25 Human Resources → /26

Executive Management → /27

As the network administrator, perform the following tasks:

- Determine the network address and broadcast address for each department.

A. Network: 10.10.0.0

- Identify the valid range of host IP addresses.

A. Broadcast: 10.10.0.255

- Calculate the number of usable hosts in each subnet.

A. Host range: 10.10.0.1- 10.10.0.254

- Identify the unused IP address range remaining within the allocated IP block.

A.UsableHosts:254

- A smart hospital is migrating its network infrastructure from IPv4 to IPv6 to support thousands of connected medical devices and IoT sensors. The hospital has been assigned the IPv6 network: 2001:0db8:abcd:0000:0000:0000:0000:0000/64

As the network engineer responsible for the migration, perform the following:

- Convert the given IPv6 address into its compressed notation.

A. Compressed Address 2001:db8:abcd::/64

- Expand	the	compressed	address	back	into	its	full	hexadecimal representation.

A. Expand Address:2001:0db8:abcd:0000:0000:0000:0000

- Identify the network prefix using the /64 prefix length.

A. Network prefix: 2001:db8:abcd::/64

- Calculate the total number of host addresses supported within the subnet.

A. Total possible Host address 18,446,744,073,709,551,616

- the total number of possible host addresses available within this network.

A. Total Possible host address 18,446,774,073,709,551,616

- An Internet Service Provider (ISP) connects multiple customer networks through a core router. The router maintains the following routing table entries: 192.168.1.0/24

192.168.2.0/24

192.168.3.0/24

A packet arrives at the router with the destination IP address:

192.168.2.45

As the routing engineer, determine:

- The destination subnet to which the packet belongs.

A. Destination subnet : 192.168.2.0/24

- The matching routing table entry.

A. Matching Route : 192.168.2.4/24

- The next-hop network/interface used to forward the packet.

A.Next Hop : interface connected to 192.168.2.0/24

- The default route that should be used if the destination does not match any routing table entry.

A. Default Route : 0.0.0.0/0

- A company establishes two office departments connected through a Layer-3 router. The allocated subnet information is as follows:

Administration LAN → 192.168.10.0/26

Finance LAN → 192.168.10.64/26

The router interfaces are configured with:

Administration Gateway → 192.168.10.1

Finance Gateway → 192.168.10.65

As the network administrator, perform the following tasks:

- Determine the network address and broadcast address for each LAN.

A. Network : 192.168.10.0/26

- Identify the valid host IP address range.

A. Broadcast : 192.168.10.63

- Suggest suitable IP addresses for three computers in each LAN.

A. Three pcs: 192.168.10.2,192.168.10.3,192.168.10.4

- Identify the default gateway that hosts in each LAN should use for communication.

A. Gateway : 192.168.10.126

RUBRICS FOR CALCULATION

| Scenario Based Assessment |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Criteria | Weightage (%) | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| Understanding of Scenario | 20% | Demonstrates clear and complete understandin g of the scenario and context | Shows good understandi ng with minor gaps | Partial understandin g; misses key aspects | Misinterpret s or fails to understand the scenario |
| Identification of Key Issues | 20% | Accurately identifies all relevant issues and factors involved | Identifies most key issues with minor omissions | Identifies limited issues; some irrelevant points | Unable to identify key issues |
| Application of Concepts | 25% | Applies appropriate concepts effectively to address the scenario | Applies concepts with minor errors | Limited or partially correct application | Unable to apply relevant concepts |
| Decision Making | 20% | Makes well-reasoned, logical decisions with strong rationale | Makes reasonable decisions with some justification | Makes weak decisions with limited justification | No clear decisions made or rationale provided |
| Clarity of Response | 15% | Response is clear, structured, and logically presented | Generally clear with minor issues | Somewhat unclear or poorly structured | Disorganize d and difficult to understand |